CODE FOR SMART HELMET:

TRANSMITTER CODE:

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <RH_ASK.h>

#include <SPI.h>

// Initialize LCD display and RF module

LiquidCrystal_I2C lcd(0x27, 16, 2);

RH_ASK rf_driver(2000, 11, 12, 0);

// Define pin connections

const int ignitionRelay = 4;

const int buzzerPin = 5;

// Variables to store received data

String str_helmet = "0", str_alcohol = "0", str_sleep = "0";

int helmetStatus = 0, alcoholLevel = 0, sleepStatus = 0;

bool engineState = false;

// Timeout and page switching variables

unsigned long lastReceivedTime = 0;

const unsigned long timeout = 5000;

int page = 0, lastPage = -1;

unsigned long lastPageSwitch = 0;

const unsigned long pageInterval = 3000;

void setup() {

Serial.begin(115200); // Initialize serial communication

rf_driver.init(); // Initialize RF module

pinMode(ignitionRelay, OUTPUT);

pinMode(buzzerPin, OUTPUT);

digitalWrite(ignitionRelay, LOW); // Default: Vehicle OFF

digitalWrite(buzzerPin, LOW); // Default: No alert

lcd.begin();

lcd.backlight();

lcd.setCursor(0, 0);

lcd.print("Smart Helmet");

delay(1000);

lcd.clear();

}

void loop() {

uint8_t buf[20];

uint8_t buflen = sizeof(buf);

// Check if RF data is received

if (rf_driver.recv(buf, &buflen)) {

String str_out = String((char*)buf);

lastReceivedTime = millis(); // Update last received time

// Extract values from received string

int comma1 = str_out.indexOf(',');

int comma2 = str_out.lastIndexOf(',');

if (comma1 > 0 && comma2 > comma1) {

str_helmet = str_out.substring(0, comma1);

str_alcohol = str_out.substring(comma1 + 1, comma2);

str_sleep = str_out.substring(comma2 + 1);

helmetStatus = str_helmet.toInt();

alcoholLevel = str_alcohol.toInt();

sleepStatus = str_sleep.toInt();

// Control vehicle ignition based on conditions

engineState = (helmetStatus && !alcoholLevel);

digitalWrite(ignitionRelay, engineState);

// Activate buzzer if sleep or alcohol detected

digitalWrite(buzzerPin, (sleepStatus || alcoholLevel || page == 99) ? HIGH : LOW);

Serial.print("Received: ");

Serial.println(str_out);

}

}

// Handle timeout if no data received for a set time

if (millis() - lastReceivedTime > timeout) {

if (page != 99) {

digitalWrite(buzzerPin, HIGH);

lcd.clear();

lcd.print("No Data Received");

lcd.setCursor(0, 1);

lcd.print("System Offline");

page = 99;

}

return;

}

// Handle automatic page switching

if (millis() - lastPageSwitch > pageInterval) {

lastPageSwitch = millis();

page = (page + 1) % 2;

delay(200); // Small delay to prevent rapid switching

}

// Clear LCD only when switching pages

if (page != lastPage) {

lcd.clear();

lastPage = page;

}

// Display different pages based on current page index

switch (page) {

case 0:  // Page 1: Helmet, Alcohol, and Sleep Status

lcd.setCursor(0, 0);

lcd.print("Helmet: ");

lcd.print(helmetStatus == 1 ? "Yes" : "No ");

lcd.setCursor(0, 1);

lcd.print("Alcohol: ");

lcd.print(alcoholLevel == 1 ? "Yes" : "No ");

break;

case 1:  // Page 2: Sleep Status and Engine State

lcd.setCursor(0, 0);

lcd.print("Sleep: ");

lcd.print(sleepStatus == 1 ? "Yes" : "No ");

lcd.setCursor(0, 1);

lcd.print("Engine: ");

lcd.print(engineState ? "ON " : "OFF");

break;

}

}

RECIVER’S CODE

#include <RH_ASK.h>

#include <SPI.h>

// Debug Mode (Set to true for debugging, false for silent mode)

#define DEBUG_MODE false

// Sensor Pins

const int alcoholSensorPin = A0;

const int wearSensorPin = A1;

const int sleepSensorPin = A2;

const int buzzerPin = 7;  // Buzzer connected to pin 7

// Create RF Object

RH_ASK rf_driver;

// Variables for drowsiness detection

const int drowsinessThreshold = 5;  // Number of sleep signals needed within time window

const unsigned long drowsinessTimeWindow = 10000; // 10 seconds window

const unsigned long resetTime = 5000;  // Reset if no sleep signals within 5 seconds

unsigned long sleepTimestamps[drowsinessThreshold];  // Stores timestamps of detected sleep signals

int sleepIndex = 0;

bool isDrowsy = false;

unsigned long lastSleepDetectionTime = 0;

unsigned long lastSendTime = 0;

const unsigned long sendInterval = 50; // Send data every 50ms

void setup() {

#if DEBUG_MODE

Serial.begin(9600);  // Start Serial Monitor

Serial.println("System Initialized...");

#endif

rf_driver.init();  // Initialize RF module

pinMode(wearSensorPin, INPUT);

pinMode(alcoholSensorPin, INPUT);

pinMode(sleepSensorPin, INPUT);

pinMode(buzzerPin, OUTPUT);

digitalWrite(buzzerPin, LOW);  // Ensure buzzer is OFF initially

}

void loop() {

unsigned long currentMillis = millis();

// Read sensor values

int alcoholLevel = !digitalRead(alcoholSensorPin);

int sleepSignal = !digitalRead(sleepSensorPin);

int helmetStatus = !digitalRead(wearSensorPin);

#if DEBUG_MODE

Serial.print("Helmet: "); Serial.print(helmetStatus);

Serial.print(" | Alcohol: "); Serial.print(alcoholLevel);

Serial.print(" | Sleep Signal: "); Serial.println(sleepSignal);

#endif

// **Drowsiness Detection Logic**

if (sleepSignal) {

lastSleepDetectionTime = currentMillis;

// Store the current timestamp in the circular buffer

sleepTimestamps[sleepIndex] = currentMillis;

sleepIndex = (sleepIndex + 1) % drowsinessThreshold;

// Check if there are 'drowsinessThreshold' sleep detections within 'drowsinessTimeWindow'

int count = 0;

for (int i = 0; i < drowsinessThreshold; i++) {

if (currentMillis - sleepTimestamps[i] <= drowsinessTimeWindow) {

count++;

}

}

isDrowsy = (count >= drowsinessThreshold);

}

// **Reset drowsiness detection if no sleep signals for resetTime**

if (currentMillis - lastSleepDetectionTime > resetTime) {

isDrowsy = false;

}

#if DEBUG_MODE

Serial.print("Drowsiness Detected: ");

Serial.println(isDrowsy ? "YES" : "NO");

#endif

// **Activate Buzzer on Drowsiness Detection**

if (isDrowsy) {

digitalWrite(buzzerPin, HIGH);  // Turn ON buzzer

} else {

digitalWrite(buzzerPin, LOW);   // Turn OFF buzzer

}

// **Send data at regular intervals**

if (currentMillis - lastSendTime >= sendInterval) {

lastSendTime = currentMillis;

// Convert sensor values to a formatted string

String data = String(helmetStatus) + "," + String(alcoholLevel) + "," + String(isDrowsy ? 1 : 0);

// Convert string to char array for RF transmission

char msg[data.length() + 1];

data.toCharArray(msg, data.length() + 1);

// Send data via RF

rf_driver.send((uint8_t *)msg, strlen(msg));

rf_driver.waitPacketSent();

#if DEBUG_MODE

Serial.print("Data Sent: ");

Serial.println(data);

#endif

}

delay(10); // Small delay to avoid overloading Serial output

}